# UFS Query Module — Interface Design

This document defines the module interface for the UFS (Universal Flash Storage) query feature in the embedded C firmware. The module abstracts host-to-device query commands defined by the UFS standard (JEDEC JESD220-series) and exposes unified read/write APIs for attributes, flags, descriptors, and extended features.

# 1. Interface List

Table 1 — UFS Query Module Interface List

| Function | Prototype | Description |
| --- | --- | --- |
| ufs_query_attr_read | int ufs_query_attr_read(uint8_t idn, uint32_t *val) | Read a UFS device attribute by IDN. |
| ufs_query_attr_write | int ufs_query_attr_write(uint8_t idn, uint32_t val) | Write a UFS device attribute by IDN. |
| ufs_query_flag_read | int ufs_query_flag_read(uint8_t idn, bool *val) | Read a UFS device flag by IDN. |
| ufs_query_flag_set | int ufs_query_flag_set(uint8_t idn) | Set a UFS device flag by IDN. |
| ufs_query_flag_clear | int ufs_query_flag_clear(uint8_t idn) | Clear a UFS device flag by IDN. |
| ufs_query_desc_read | int ufs_query_desc_read(uint8_t type, uint8_t idx, uint8_t *buf, uint16_t *len) | Read a UFS descriptor (Device, Geometry, Unit, etc.). |
| ufs_query_ext_feat | int ufs_query_ext_feat(uint8_t type, uint8_t sub_type, uint32_t *val) | Read an extended feature parameter. |
| ufs_query_req_send | int ufs_query_req_send(uint8_t *upiu, uint16_t upiu_len, uint8_t *resp, uint16_t *resp_len) | Send raw query request UPIU and receive response. |

# 2. Parameter Table

Table 2 — Parameter Definitions

| Parameter | Direction | Type | Range | Description |
| --- | --- | --- | --- | --- |
| idn | IN | uint8_t | 0x00–0xFF | Identifier number for attribute, flag, or extended feature. |
| val | OUT | uint32_t * | N/A | Pointer to store the read attribute/feature value. |
| val | IN | uint32_t | 0–0xFFFFFFFF | Attribute value to write. |
| val | OUT | bool * | N/A | Pointer to store the read flag state (true/false). |
| type | IN | uint8_t | 0x00–0xFF | Descriptor type code (e.g. 0x00 = Device, 0x01 = Geometry). |
| idx | IN | uint8_t | 0x00–0xFF | Descriptor index for multi-instance descriptors. |
| buf | OUT | uint8_t * | N/A | Pointer to descriptor data buffer. |
| len | IN/OUT | uint16_t * | 0–65535 | Input: buffer capacity; output: bytes written. |
| sub_type | IN | uint8_t | 0x00–0xFF | Extended feature sub-type selector. |
| upiu | IN | uint8_t * | N/A | Pointer to raw query request UPIU buffer. |
| upiu_len | IN | uint16_t | 0–65535 | Length of the query request UPIU. |
| resp | OUT | uint8_t * | N/A | Pointer to response UPIU buffer. |
| resp_len | IN/OUT | uint16_t * | 0–65535 | Input: response buffer capacity; output: received length. |

# 3. Error Code Table

Table 3 — Error Codes

| Macro | Value | Description |
| --- | --- | --- |
| UFS_OK | 0 | Operation completed successfully. |
| UFS_ERR_GENERAL | -1 | Unspecified or generic failure. |
| UFS_ERR_TIMEOUT | -2 | Query request timed out waiting for device response. |
| UFS_ERR_NACK | -3 | Device returned negative acknowledgment (NACK). |
| UFS_ERR_INVAL | -4 | Invalid parameter (null pointer, out-of-range IDN, etc.). |
| UFS_ERR_BUSY | -5 | Device or UIC layer busy; retry later. |
| UFS_ERR_NOTSUPP | -6 | Requested IDN or type not supported by the device. |
| UFS_ERR_CRC | -7 | CRC mismatch in query response frame. |
| UFS_ERR_OVERRUN | -8 | Descriptor buffer too small for device response. |

# 4. Notes

All APIs are blocking and run on the UFS task context; callers must not invoke them from interrupt handlers.

The query transport uses the UFS native Query Request UPIU (UPIU = UFS Protocol Information Unit) defined in JEDEC JESD223. The module transparently handles UPIU assembly, command submission, and response parsing.

A 5-second timeout is applied per query transaction. If the device does not respond within this window, the command is aborted and UFS_ERR_TIMEOUT is returned.

The descriptor read API (ufs_query_desc_read) accepts a buffer capacity via *len. On return, *len is updated to the actual descriptor length. If the buffer is too small, UFS_ERR_OVERRUN is returned and *len reflects the required size.

The module serialises all query requests through a single mutex-guarded channel. Concurrent calls block on the mutex and are serviced in FIFO order.

Error code values are negative integers; UFS_OK (0) is the only success code. Callers should test ret == UFS_OK, not !ret.

Extended feature queries (ufs_query_ext_feat) are only available on UFS 3.1+ devices that advertise ExtendedFeatures in the Geometry descriptor.

All IDN, type, and sub_type constants are defined in ufs_spec_defs.h and follow JEDEC-assigned numbering.

--- End of Document ---